Prompt Radiation Utility vs. Excel Template Comparison

Comparison of new Qt utility output with PromptRadiationTemplate_Expanded_FY2027 Excel calculation

# 1. Executive summary

Main result: The new utility gives N0318 = 21.49 mrem/h, while the Excel template gives approximately 19.3 mrem/h. The utility is higher by 2.19 mrem/h, or +11.4%. This is a reasonable first comparison for the intermediate text-file version, but it is not yet a bit-for-bit reproduction of Excel.

Most likely cause of the N0318 difference: the low-ion block used by the utility is not the same as the low-ion block shown in Excel. In the utility output, He-4 low and H-3 low alone contribute about 5.26 mrem/h to N0318. In the Excel pasted low-ion block, the low-ion subtotal is about 1.6 mrem/h. This difference can explain most of the N0318 offset.

Secondary issue: some monitors with large P5 sensitivity, especially N0314, N0305, and N0308B, are approximately 29-30% lower in the utility. This suggests that the detailed isotope rows, low-ion locations, or DB0.x location assignment should be checked carefully against the Excel sheet.

| Item | Utility | Excel / observation | Interpretation |
| --- | --- | --- | --- |
| N0318 total | 21.494 mrem/h | 19.3 mrem/h | +2.194 mrem/h (+11.4%) |
| Largest absolute differences | N0314, N0305, N0308B | Excel is higher | Utility is about 70% of Excel for these monitors |
| Largest likely source of N0318 mismatch | Low-ion block | Utility low-ion assumptions differ from Excel | Align low-ion rows first |
| Input-file run status | 70 yield rows accepted/skipped 27; 97 position rows accepted/skipped 0; 60 detailed + 5 low-ion after cutoff | From utility output | Check whether skipped yield rows are expected |

# 2. Monitor total comparison

Excel totals were copied from the spreadsheet in scientific notation and appear rounded to approximately three significant digits. Therefore, very small differences should not be over-interpreted; the important differences are the systematic 10-30% shifts.

| Monitor | Utility mrem/h | Excel mrem/h | Utility - Excel | Difference % | Utility / Excel | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| N0318 | 21.4941 | 19.3 | +2.19415 | +11.4% | 1.114 | Utility higher |
| N0306 | 0.823863 | 1.05 | -0.226137 | -21.5% | 0.785 | Utility lower |
| N0319 | 1.11408 | 1.57 | -0.455918 | -29.0% | 0.710 | Utility lower |
| N0325A | 121.563 | 114 | +7.56256 | +6.6% | 1.066 | Good agreement |
| N0325B | 16.2655 | 15.1 | +1.16549 | +7.7% | 1.077 | Good agreement |
| N0304B | 44.3522 | 52.7 | -8.34781 | -15.8% | 0.842 | Utility lower |
| N0314 | 1331.43 | 1900 | -568.569 | -29.9% | 0.701 | Utility lower |
| N0305 | 1331.43 | 1900 | -568.569 | -29.9% | 0.701 | Utility lower |
| N0308B | 744.844 | 1050 | -305.156 | -29.1% | 0.709 | Utility lower |
| N0322 | 0.813008 | 0.921 | -0.107992 | -11.7% | 0.883 | Utility lower |
| N0323 | 1.02918 | 1.12 | -0.0908195 | -8.1% | 0.919 | Good agreement |
| N0324 | 0.0977429 | 0.0806 | +0.0171429 | +21.3% | 1.213 | Utility higher |
| N0304 | 0.0694394 | 0.0984 | -0.0289606 | -29.4% | 0.706 | Utility lower |
| N0315 | 0.181308 | 0.15 | +0.0313082 | +20.9% | 1.209 | Utility higher |

# 3. Top N0318 contributors from the utility output

The first two utility contributors are low-ion rows. Their combined contribution is about 5.26 mrem/h, which is already larger than the complete Excel low-ion subtotal shown in the pasted table.

| Rank | Fragment | Location | DB0.x [mm] | Rate [pps] | N0318 [mrem/h] |
| --- | --- | --- | --- | --- | --- |
| 1 | He-4 low | P2 | -- | 2.35e+09 | 2.91665 |
| 2 | H-3 low | P2 | -- | 1.01e+09 | 2.3435 |
| 3 | 36P | P2 | -- | 5.56e+08 | 1.65622 |
| 4 | 28Mg | P4 | -140.73 | 2.82e+08 | 1.27688 |
| 5 | 26Na | P4 | -168.16 | 2.64e+08 | 1.20704 |
| 6 | 23Ne | P3 | -232.27 | 2.39e+08 | 1.08217 |
| 7 | 21F | P3 | -262.66 | 2.05e+08 | 0.932121 |
| 8 | 34Si | P2 | -- | 2.58e+08 | 0.779347 |
| 9 | 25Na | P3 | -203.66 | 1.65e+08 | 0.743427 |
| 10 | 24Ne | P3 | -197.69 | 1.42e+08 | 0.705502 |

# 4. Low-ion block comparison

Important observation: The utility and Excel are not using the same low-ion rows. This is the clearest explanation for the N0318 difference. Before checking deeper formulas, make the low-ion rows identical in both calculations.

Excel low-ion block visible in the pasted table:

| Ion | A | Z | pps | MeV/u | Scale | Loc | N0318 factor | N0318 mrem/h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H-3 low | 3 | 1 | 6.5e+08 | 120 | 0.524 | P2 | 0.16 | 0.544 |
| H-2 low | 2 | 1 | 8.5e+08 | 80 | 0.117 | P3 | 0.29 | 0.288 |
| H-1 low | 1 | 1 | 3e+09 | 100 | 0.0427 | P2 | 0.16 | 0.205 |
| He-4 low | 4 | 2 | 2e+08 | 200 | 0.682 | P3 | 0.29 | 0.396 |

Utility low-ion assumptions currently used or implied by the output:

| Ion | A | Z | pps | MeV/u | Loc | N0318 mrem/h |
| --- | --- | --- | --- | --- | --- | --- |
| H-3 low | 3 | 1 | 1.01e+09 | 218.51 | P2 | 2.3435 |
| H-2 low | 2 | 1 | 1.63e+09 | 223.4 | P5 | ~0.16 |
| He-4 low | 4 | 2 | 2.35e+09 | 215.65 | P2 | 2.91665 |
| H-1 low | 1 | 1 | 0 | 0 | P4 | 0 |
| He-3 low | 3 | 2 | 7.5e+07 | 214.63 | P3 | ~0.09 |

Low-ion conclusion: Excel low-ion subtotal is about 1.6 mrem/h for N0318. The utility low-ion block is approximately 5.5 mrem/h for N0318 using the current default rows. The difference is about +3.9 mrem/h, larger than the total N0318 utility-vs-Excel difference of +2.19 mrem/h. That means the low-ion mismatch is dominant, but the detailed non-low rows are probably lower in the utility by roughly 1.7 mrem/h and should also be checked.

# 5. Conclusions and recommended checks

| Priority | Recommendation |
| --- | --- |
| Do first | Make the utility low-ion rows identical to the Excel low-ion block. Use the same rates, energies, locations, and light-ion multiplier. Then rerun the comparison. |
| Check DB0.x matching | Several top contributors have DB0.x shown as "--" and therefore fall back to P2. Confirm this matches the Excel VLOOKUP behavior and not a fragment-name mismatch. |
| Check skipped rows | The utility reports 70/27 yield rows accepted/skipped, but only 60 detailed rows after cutoff. Confirm whether the 27 skipped rows are blank/invalid and whether the 10 removed by cutoff are expected. |
| Check P5-sensitive monitors | N0314, N0305, and N0308B are about 29-30% lower than Excel. Because these monitors have very large P5 factors, small differences in P5 assignment or low-ion location can produce large differences. |
| After alignment | A good target for this intermediate utility is agreement within a few percent for N0318 and comparable agreement for all monitor totals, limited by Excel rounding and any intentional changes in assumptions. |

Prepared from the utility output and Excel values pasted in the comparison request. Excel totals are rounded as pasted; use exact cell values from the workbook for final numerical validation.